SNACOM 3172 LAN GATEWAY for VTAM

------ ---- --- ------- --- ----

SNACOM 3172 Managers for VTAM are a repackaging of the SNACOM 3172 code for

a P/390 environment and like the real 3172 has three base components:

�����������������������������������������������������������������������������Ŀ

� SNA LAN GATEWAY - LSApLAN/LAN3172                                           �

� --- --- -------                                                             �

�   provides the base Link Services Architecture (LSA) PU Type 2.1 Peripheral �

�   and PU Type 4/5 SubArea LAN 802.2 InterConnect function thru the VTAM XCA �

�   MajorNode.                                                                �

�������������������������������������������������������������������������������

SNA BOX MANAGER - LSApBMGR/MGR3172

--- --- -------

provides the NetView BoxManager function for use with NetView.

SNA SDLC GATEWAY - LSApSDLC/SDL3172

--- ---- -------

adds limited Integrated Communication Adapter (ICA) capability to the

VTAM XCA MajorNode by an under-the-cover mapping of LSA LAN Adapters

to SDLC (ARTIC/PortMaster or Wide Area Connector (WAC)) lines.

intent is to provide SDLC connections to MVS VTAM which would otherwise

require a 37x5 NCP/PEP (unlike VM or VSE, MVS VTAM never supported an

ICA).

DEVICE TYPE - 3088

------ ----

�����������������������

�                     �

� P/390 CONFIGURATION �

�                     �

�����������������������

on a 3172, SNACOM maps to four consecutive S/390 SubChannel (CCUU)

addresses (only the base address is specified in the configuration) ...

assuming a base Channel/Unit of 280 the 3172 convention is:

0280  NetView BoxManager

0281  LAN GateWay

0282  SDLC GateWay Adapter0 (SDLC lines 00:07)

0283  SDLC GateWay Adapter1 (SDLC lines 08:15)

for SNACOM 3172 Managers for VTAM, the four P/390 SubChannel addresses are

mapped by ManagerName and need not be consecutive.

also, SNACOM 3172 LAN GateWay for VTAM allows multiple SNA/LSA InterConnect

SubChannel addresses.

to configure, go to the P/390 Configuration "Update System Devices" (F2)

screen and configure the 3088 SubChannel addresses.  Use ManagerNames

from the following table depending upon the function required (you need

only configure the ManagerNames for functions you want) ...

������������������������������������������������������͸

� SNACOM Function       � ManagerName  � ProgramName*  �

������������������������������������������������������;

������������������������������������������������������͸

� LAN (802.2) GateWay   � LAN3172      � LSAPLAN.EXE   �

������������������������������������������������������;

� SDLC GateWay          � WAN3172      � LSAPSDLC.EXE  �

������������������������������������������������������Ĵ

� NetView BoxManager    � MGR3172      � LSAPBMGR.EXE  �

��������������������������������������������������������

* ProgramName is the name of the .EXE file used in the IPL.CMD file

you may use the FN/P field as a comment field (the FN/P field is not used

by the SNACOM 3172 Emulator for VTAM).

EXAMPLE:

-------

on the P/390 Configuration "Update System Devices" (F2) screen ...

������������������������������ Available disk space in Mbytes: 36 ������������ͻ

�Addr Device  Label  Atype  Size    Mgr   FN/P     PATH:>E:\                  <�

���Ŀ����Ŀ-------------------------�Ŀ----------------------------------------�

�280��3088� >       >     >         �9�>                                       �

�����������                         ���                                        �

�                                                                              �

������������������������������������������������������������������������������ͼ

Mgr Codes: 1=AWSFBA   2=AWSCKD   3=AWS3274  4=LAN3274  5=AWS3215  6=AWS2821

7=AWS2540  8=LAN3088  9=LAN3172  A=LCS3172  B=MGR3172  C=WAN3172  D=AWS2703

E=AWSICA   F=AWSPBS   G=AWSARX   H=AWSTAPE  I=SCSI3480 J=SCSI3420 K=AWSOMA

L=AWS9346  M=AWSTFA   N=AWSPCSRV O=CHAN370

��������������������������������������������������������������������������������

F1 Help  ALT+F1 Key Definitions  F10 Main Menu  ESC Cancel Input   � cnsysdev

�����Ŀ

enter the values at the �boxes� where ...

�������

���Ŀ                               �Ŀ

�280� is S/390 SubChannel address   �9� is LAN3172 (LAN GateWay) MgrCode

�����                               ���

would result in the following entry for a LAN GateWay SubChannel

optional comments may  ����������������������Ŀ

be added now or later  �3172 LAN (802) GateWay�

������������������������

������������������������������ Available disk space in Mbytes: 36 ������������ͻ

�Addr Device  Label  Atype  Size    Mgr   FN/P     PATH:>E:\                  <�

�------------------------------------------------------------------------------�

>    |      >       >     >         >  >                                       �

�                                                                              �

����������������������Ŀ

�280  3088                           9 �3172 LAN (802) GateWay�                �

������������������������

�                                                                              �

������������������������������������������������������������������������������ͼ

Mgr Codes: 1=AWSFBA   2=AWSCKD   3=AWS3274  4=LAN3274  5=AWS3215  6=AWS2821

7=AWS2540  8=LAN3088  9=LAN3172  A=LCS3172  B=MGR3172  C=WAN3172  D=AWS2703

E=AWSICA   F=AWSPBS   G=AWSARX   H=AWSTAPE  I=SCSI3480 J=SCSI3420 K=AWSOMA

L=AWS9346  M=AWSTFA   N=AWSPCSRV O=CHAN370

��������������������������������������������������������������������������������

F1 Help  ALT+F1 Key Definitions  F10 Main Menu  ESC Cancel Input   � cnsysdev

multiple SNA/LSA SubChannel addresses can be specified and multiple LAN

Adapters are supported through a single SNA/LSA address.

if multiple SNA/LSA SubChannels are used concurrently with the same LAN

Adapter, then each XCA MajorNode (using each SNA/LSA SubChannel) must use

a different SAP (a SAP is defined below in section 802.2 DLC Parameters in

topic LAPS Configuration).

for TCP/IP use the LCS3172 device manager.

the SNACOM 3172 Emulator for VTAM can coexist with LCS3172 on the same

LAN Adapter.

INSTALLATION NOTES:

------------ -----

�� HARDWARE LIMITATIONS ��������������������������������������������������Ŀ

�                                                                          �

�    FOR SERVER/330 WITH BUILTIN AMD 10MBIT ETHERNET ADAPTER, DO NOT USE   �

�    ANY OF THE IBM PCI ETHERNET ADAPTERS THAT USE THE IBMEANDI.OS2 NDIS   �

�    DRIVER (IBM PCI ETHERNET ADAPTER 13H9237) SINCE THE IBMEANDI.OS2      �

�    NDIS DRIVER WILL NOT CO-EXIST WITH THE PCNTND.OS2 DRIVER USED BY      �

�    THE SERVER/330 BUILT-IN 10MBIT ETHERNET ADAPTER                       �

�                                                                          �

�    DO NOT USE THE 3COM EL90X.OS2 NDIS DRIVER (3COM ETHERLINK XL 3C905    �

�    10/100 PCI ADAPTER) SINCE THE EL90X.OS2 DRIVER (AT LEAST THRU         �

�    RELEASE 3.0F 01/23/1998) IS SUBJECT TO DEVICE DRIVER TRAPS            �

�                                                                          �

����������������������������������������������������������������������������

Normally, these two files will contain run time messages:

AWS3172.LOG - contains Informational Messages

AWS3172.ERR - contains Error Message

For debug and trace entries, add a parameter to the device manager, using

the P/390 Configurator "Update System Devices" menu, "Device Manager Parm

Edit/Display" submenu (F2/F12).

LAN3172   > Debug(#)

where # = 0 = No tracing (default)

# = 1 = Summary tracing

# = 2 = Detailed tracing

The SNA LAN Gateway (LSApLAN) has one optional program parameter to

override the default SAP Receive Buffer Size.  The default for the SAP

Receive Buffer Size is 640 bytes.  This traditional default along with

the NDIS requirement that a received frame span at most eight SAP Receive

Buffers results in a maximum receive frame length of about 5000 bytes

(8*640=5120) when buffer overhead requirements are subtracted.

The Receive Buffer Size may be specified for all LAN Adapters (/RB=) or

by LAN Adapter (/RB#=) where # specifies the LAPS LAN Adpater number

in hex (0-9, A-F).

Use P/390 Configurator "Update System Devices" menu, "Device Manager Parm

Edit/Display" submenu (F2/F12) to change these parameters:

LAN3172    > /RB=1280 /RB1=640

where /RB=1280 establishes a global default for all LAN Adapters except

for LAN Adapter1 which will use a ReceiveBufferSize of 640 bytes.  The

ReceiveBufferSize actually used for each LAN adapter may be confirmed

by viewing the LSApLAN entries in AWS3172.LOG ...

������������������������������������������������������������Ŀ

�   3172 SNA Communications Program Version 1.0, LAN support �

�                                                            �

�   Configured LAN adapter numbers and VTAM type:            �

�           0    RING  ReceiveBufferSize=01280               �

�           1  CSMACD  ReceiveBufferSize=00640               �

�                                                            �

�   3172 SNA Communications Program, LAN support running     �

��������������������������������������������������������������

Note that the total SAP Receive Buffer Pool is 64000 bytes so that a SAP

Receive Buffer Size of 640 would result in 100 SAP buffers (and 1280 would

result in 50 SAP buffers and the ability to receive a frame about 10000

bytes in length).  A SAP Receive Buffer Size of 2560 would be required to

receive the maximum architected TokenRing frame (about 18000 bytes).  Even

though all of the SAP Receive Buffer Sizes used here as examples divide

evenly into 64000, that does not have to be the case.

����������������������  ���������������������

�                    �  �                   �

� LAPS CONFIGURATION �  � LAN/802.2 LSApLAN �

�                    �  �                   �

����������������������  ���������������������

To configure LAPS, change to the \IBMCOM directory and enter "LAPS"

or select the LAPS Icon (placed by default in the System Setup Folder

in the System Folder when LAPS is installed).

In the "Configure Workstation" window, select the entry in the

"Network Adapters" list that matches your adapter card.  If there is

no matching entry, install the appropriate device manager from the

diskette that came with the adapter (exit LAPS and after re-invoking

LAPS use the Install function on the initial LAPS screen).  Any

adapter having a LAPS compatible NDIS interface may be used.  You may

have to manually install the adapter into PROTOCOL.INI and CONFIG.SYS

if the adapter support diskette is not in the format expected by the

LAPS installation utility.

If "IBM IEEE 802.2" is not listed with the adapter in the "Current

Configuration" window, then use the "Add" button to add "IBM IEEE 802.2"

from the "Protocols" window to the "Current Configuration" window.

The LAPS defaults may be insufficient to support your configuration.

In the "Current Configuration" window following the adapter to be used

select "IBM IEEE 802.2" and use the "Edit" button to review the 802.2 DLC

parameters.  Note that the protocols are grouped and listed following the

adapter to which they apply.  Before each protocol listed in the "Current

Configuration" box is a number which is the LAPS number of the adapter to

which they belong.  This LAPS number is used in the VTAM XCA MajorNode

(ADAPTER=) to identify the LAPS adapter to use.  LAPS numbers, beginning

with zero, are consecutively assigned to adapters in the order in which

the adapters are configured to LAPS.

NETWORK ADDRESS:

------- -------

Before evaluating the 802.2 DLC parameters, consider what network address

to use for the LAPS LAN adapter.  Keep in mind that the network address

of the LAPS LAN adapter will be a part of the link definitions in all of

the devices in communication with the P/390.  Unless you are prepared to

change all of these definitions whenever the physical LAPS LAN adapter

changes, you should choose a Locally Administered Address for the LAPS

LAN adapter (since a Universal or "Burned-In" network address will change

if the LAPS LAN adapter is replaced).

In the "Current Configuration" window select the LAPS LAN adapter name

and use the "Edit" button to review adapter parameters.  If the "Network

Adapter Address" field  contains an address of the form 4000xxxxxxxx, then

take note of this Locally Administered Address for use in defining remote

connections.  If the "Network Adapter Address" is blank and you want to use

a Locally Administered Address, then choose an address of the form

4000xxxxxxxx such that the resultant address is unique in your network.

For example, the "Network Adapter Address" 400074900001 matches the P/390

VM Starter System node "SYS1".  Leaving "Network Adapter Address" blank

instructs LAPS to use the Universal Address.

��������������������������������������������������������������������������Ŀ

� NOTE:  for TokenRing Adapters the address is entered in NonCanonical     �

�         (TokenRing) format (400000000000-7FFFFFFFFFFF)                   �

�                                                                          �

�        for EtherNet Adapters the address is entered in Canonical         �

�        (ByteReversed) format (enter 0200xxxxxxxxx for 4000yyyyyyyy)      �

�        In general, the other (NonCanonical or TokenRing) format of       �

�        this address will be used in the remote link definitions.         �

����������������������������������������������������������������������������

802.2 DLC PARAMETERS:

----- --- ----------

Some IBM IEEE 802.2 DLC parameters may need to be changed.  If applications

in addition to SNACOM 3172 Emulator for VTAM will be concurrently using the

LAPS LAN adapter, then add the SNACOM 3172 Emulator for VTAM values to the

requirements of the other applications to get the total system-wide values

for LAPS.

In the "Current Configuration" window select the "IBM IEEE 802.2" item under

the LAPS LAN adapter and use the "Edit" button to review the following

parameters:

������������������������Ŀ

�  Maximum Link Stations �  DEFAULT=8  MAXIMUM=255

��������������������������

This parameter specifies the maximum number of link stations that can

concurrently exist for all SAPs.  Link stations form the end points of

logical connections.  Increase this parameter by the number of link

stations implied by the sum of the VTAM XCA MajorNode definitions:

REQUIREMENT: each LINE/PU pair in the VTAM XCA MajorNode (VBUILD

TYPE=XCA) requires one link station.

link stations are allocated to the XCA SAP when VTAM

activates the XCA MajorNode.

the following XCA definition requires five link stations:

���������������������������������������������������������������������Ŀ

� NGXCA VBUILD  TYPE=XCA                                              �

�                                                                     �

� NGX280E  PORT CUADDR=280,ADAPNO=1,SAPADDR=4,MEDIUM=CSMACD,          �

�               DELAY=0.1,TIMER=20                                    �

�                                                                     �

| *------------------------------------------- NGXCA PERIPHERAL NODES �

� *------------------------------------------------------------------ �

�                                                                     �

� NGXPNGE GROUP DIAL=YES,CALL=INOUT,ANSWER=ON,ISTATUS=ACTIVE          �

�                                                                     �

� NGXENL0 LINE #��������� each LINE/PU pair requires one              �

� NGXENP0 PU              dedicated LAPS 802.2 link station           �

�                                                                     �

� NGXENL1 LINE                                                        �

� NGXENP1 PU                                                          �

�                                                                     �

� NGXENL2 LINE                                                        �

� NGXENP2 PU                                                          �

�                                                                     �

� NGXENL3 LINE                                                        �

� NGXENP3 PU                                                          �

�                                                                     �

� NGXENL4 LINE                                                        �

� NGXENP4 PU                                                          �

�����������������������������������������������������������������������

All of the link stations are allocated to the XCA SAP when VTAM opens

the XCA MajorNode.  If SNACOM 3172 Emulator for VTAM is unable to

acquire the number of link stations requested by VTAM (based upon XCA

MajorNode definitions), then the XCA MajorNode will fail to activate

(VTAM message IST380I with SENSE:081C0000) and a message (XCA0043 with

RC=46) will be written to AWS3172.ERR:

������������������������������������������������������������������������Ŀ

� IST380I ERROR FOR ID - NGAXSNL - REQUEST:ACTLINK, SENSE:081C0000       �

�                                                                        �

�                                                                        �

� XCA043: A FAILURE OCCURRED IN OPENING A SAP: RC 46 SAP-VALUE 4         �

�                                                                        �

�                           (RC 46 = inadequate number of link stations) �

��������������������������������������������������������������������������

Because the CCB Return Code ('46'x) for DLC.OPEN.SAP is used for either

inadequate link stations or SAPs, these error messages cannot be used

to distinguish between either a link station or a SAP shortage or both.

���������������Ŀ

�  Maximum SAPs �  DEFAULT=3  MAXIMUM=126

�����������������

This parameter specifies the maximum number of Service Access Points

(SAPs) that can concurrently exist.  One ore more SAPs are assigned to

an application and provide a mechanism for sharing a single LAN adapter

(and single network address) by multiple applications.  The SAP can be

considered as part of the network adapter address (and is sometimes

specified that way) with the two digit SAP address and the 12 digit

network address forming a 14 digit network address.

REQUIREMENT: one (1) SAP per SubChannel

For two 802.2 applications to communicate, each must have an active SAP.

If CM/2 is to be used on the P/390 to provide "NonDFT" LAN sessions to

the P/390, then two SAPs will be required (one for LAN3172 and one for

CM/2).

the following application mix would require 4 SAPs:

. CM/2 LAN (3270 emulation)

. CM/2 LAN (APPC)

. MGR3172

. LAN3172

If SNACOM 3172 Emulator for VTAM is unable to acquire a SAP because that

action would cause the maximum number of SAPs to be exceeded, then XCA

MajorNode will fail to activate (VTAM message IST380I with SENSE:081C0000)

and a message (XCA0043 with RC=46) will be written to AWS3172.ERR:

������������������������������������������������������������������������Ŀ

� IST380I ERROR FOR ID - NGAXSNL - REQUEST:ACTLINK, SENSE:081C0000       �

�                                                                        �

�                                                                        �

� XCA043: A FAILURE OCCURRED IN OPENING A SAP: RC 46 SAP-VALUE 4         �

�                                                                        �

�                                    (RC 46 = inadequate number of SAPs) �

��������������������������������������������������������������������������

Because the CCB Return Code ('46'x) for DLC.OPEN.SAP is used for either

inadequate link stations or SAPs, these error messages cannot be used

to distinguish between either a link station or a SAP shortage or both.

SNACOM 3172 Emulator for VTAM uses the SAP defined by VTAM XCA MajorNode

SAPADDR= parameter.

���������������������������������������������������������������������Ŀ

� NGXCA VBUILD  TYPE=XCA                                              �

�                                                                     �

� NGX280E  PORT CUADDR=280,ADAPNO=1,SAPADDR=4,MEDIUM=CSMACD,          �

�               DELAY=0.1,TIMER=20                                    �

�����������������������������������������������������������������������

Consideration needs to be given to the SAP value used by SNACOM 3172

Emulator for VTAM.  SAPs range in value from '04'x to 'FC'x by '04'x.

SAP('04'x) used to be considered the "SNA SAP" and, as a result,

SAP('04'x) was often assumed by products using 802.2 protocols to

communicate with VTAM.  Some older 3270 terminal emulators may not

allow a SAP('04'x) default to be overridden, so LAN3172 must use

SAP('04'x) if LAN3172 is to communicate with one of these emulators.

Since CM/2 defaults to SAP('04'x) for 802.2 ("NonDFT') 3270 emulation,

the recommendation is to change the CM/2 local SAP to something other

than '04'x and use SAP('04'x) for LAN3172 (in CM/2 setup choose

"Token-ring or other LAN types" in the "Workstation Connection Type"

box and "3270 emulation" in the "Feature or Application" box and then

select the "DLC - Token-ring or other LAN types" line).

If SNACOM 3172 Emulator for VTAM is unable to acquire SAP(SAPADDR=),

then the XCA MajorNode will fail to activate (VTAM message IST380I

with SENSE:081C0000) and an error message (XCA043 with RC=43) will

be written to AWS3172.ERR:

������������������������������������������������������������������������Ŀ

� IST380I ERROR FOR ID - NGAXSNL - REQUEST:ACTLINK, SENSE:081C0000       �

�                                                                        �

�                                                                        �

� XCA043: A FAILURE OCCURRED IN OPENING A SAP: RC 43 SAP-VALUE 4         �

�                                                                        �

�                      (RC 43 = invalid SAP value or SAP already in use) �

��������������������������������������������������������������������������

��������������������������Ŀ

�  Maximum Number of Users �  DEFAULT=3  MAXIMUM=12

����������������������������

This parameter specifies the maximum number of applications that can

concurrently use the protocol.

REQUIREMENT: one (1) user per SNACOM function (MGR3172+LAN3172+WAN3172)

If SNACOM 3172 Emulator for VTAM is unable to open the LAPS LAN adapter

because that action would exceed the user limit, then the XCA MajorNode

will fail to activate  (VTAM message IST380I with SENSE:081C0000) and

a message (XCA007 with RC=61) will be written to AWS3172.ERR:

������������������������������������������������������������������������Ŀ

� IST380I ERROR FOR ID - NGAXSNL - REQUEST:ACTLINK, SENSE:081C0000       �

�                                                                        �

�                                                                        �

� XCA007: ERROR OCCURRED WHILE OPENING LAN ADAPTER: RC 61 ERROR-CODE 0   �

�                                                                        �

�                      (RC 61 = number of application programs exceeded) �

��������������������������������������������������������������������������

��������������������������������������������������������������������������Ŀ

�                                                                          �

�           NOTE ABOUT LAPS 802.2 RESOURCE DEFINITION PROBLEMS             �

�                                                                          �

��������������������������������������������������������������������������Ķ

�                                                                          �

�   Problems arising from incorrect/inadequate 802.2 definitions occur     �

�   when VTAM activates XCA MajorNode if the Emulator is unable to open    �

�   the LAN adapter with the parameters supplied by VTAM.  VTAM becomes    �

�   aware of most of these errors when the VTAM request times out with     �

�   (message IST380I with SENSE:081C0000).  TIMER= value XCA MajorNode     �

�   parameter governs the timeout for these errors:                        �

�                                                                          �

�     NGX280R  PORT CUADDR=280,ADAPNO=1,SAPADDR=4,MEDIUM=RING,             �

�                   DELAY=0.1,TIMER=20                                     �

�                                                                          �

�   If the XCA MajorNode remains active for a period of time exceeding     �

�   the TIMER= value, then use that as an indication that an LAPS 802.2    �

�   connection has been established and the LAPS 802.2 configuration       �

�   is adequate for operation (only likely remaining problem would be      �

�   with the actual value of the "Network Adapter Address" not being       �

�   correct on an EtherNet due to confusion arising from its Canonical     �

�   representation).                                                       �

�                                                                          �

��������������������������������������������������������������������������Ķ

�                                                                          �

�   There are tools available to dynamically list the LAPS Adapter control �

�   blocks which will show the LAPS resources and how they are allocated.  �

�                                                                          �

�   One such program, LAPSDUMP, is available from P/390 support if you are �

�   having difficulty establishing your P/390 LAPS 802.2 configuration.    �

�                                                                          �

��������������������������������������������������������������������������Ľ

����������������������  ���������������������

�                    �  �                   �

� VTAM CONFIGURATION �  � LAN/802.2 LSApLAN �

�                    �  �                   �

����������������������  ���������������������

There are two types of connections possible with VTAM using LAN3172

(or WAN3172):

1) SUBAREA CONNECTIONS: between TYPE4 (NCP) or TYPE5 (SSCP) nodes which

are defined to VTAM as "LEASED" lines following

a GROUP DIAL=NO statement

2) PERIPHERAL CONNECTIONS: between TYPE2 nodes which are defined to VTAM

as "SWITCHED" lines following a GROUP DIAL=YES

statement

Consideration needs to be given to the SAP value used by SNACOM 3172

Emulator for VTAM.  SAPs range in value from '04'x to 'FC'x by '04'x.

SAP('04'x) used to be considered the "SNA SAP" and, as a result,

SAP('04'x) was often assumed by products using 802.2 protocols to

communicate with VTAM.  Some older 3270 terminal emulators may not

allow a SAP('04'x) default to be overridden, so LAN3172 must use

SAP('04'x) if LAN3172 is to communicate with one of these emulators.

Since CM/2 defaults to SAP('04'x) for 802.2 ("NonDFT') 3270 emulation,

the recommendation is to change the CM/2 local SAP to something other

than '04'x and use SAP('04'x) for LAN3172 (in CM/2 setup choose

"Token-ring or other LAN types" in the "Workstation Connection Type"

box and "3270 emulation" in the "Feature or Application" box and then

select the "DLC - Token-ring or other LAN types" line).

If SNACOM 3172 Emulator for VTAM is unable to acquire SAP(SAPADDR=),

then the XCA MajorNode will fail to activate (VTAM message IST380I

with SENSE:081C0000) and an error message (XCA043 with RC=43) will

be written to AWS3172.ERR:

������������������������������������������������������������������������Ŀ

� IST380I ERROR FOR ID - NGAXSNL - REQUEST:ACTLINK, SENSE:081C0000       �

�                                                                        �

�                                                                        �

� XCA043: A FAILURE OCCURRED IN OPENING A SAP: RC 43 SAP-VALUE 4         �

�                                                                        �

�                      (RC 43 = invalid SAP value or SAP already in use) �

��������������������������������������������������������������������������

����������������������������Ŀ

�Ĵ VTAM XCA TO XCA CONNECTION �������������������������������������������Ŀ

� ������������������������������                                           �

�                                                                          �

�                                                                          �

�                       �����������Ŀ                                      �

�  ��������������Ŀ     �           �     ��������������Ŀ                 �

�  � VMA   SA=193 �����Ĵ TokenRing �����Ĵ VMB   SA=194 �                 �

�  ����������������     �           �     ����������������                 �

�    4000749000C1       �������������       4000749000C2                   �

�                                                                          �

�                                                                          �

� ����������������������Ŀ                                                 �

�Ĵ VMA � XCA MAJOR NODE �������������������������������������������������Ĵ

� ������������������������                                                 �

� NGAXCA VBUILD  TYPE=XCA                                                  �

� *                                                                        �

� NGAX280 PORT   CUADDR=280,ADAPNO=0,SAPADDR=4,MEDIUM=RING,              - �

|                DELAY=0.1,TIMER=20                                        �

� *                                                                        �

� *------------------------------------------------ NGAXCA SUBAREA NODES   �

� *---------------------------------------------------------------------   �

� *                                                                        �

� *--------------------------------------------------------- SUBAREA VMB   �

� NGAXSNGB GROUP DIAL=NO                                                   �

� *              ----------------------------------------------- LAN3172   �

� NGAXSNLB  LINE USER=SNA,ISTATUS=ACTIVE                                   �

� *                                                                        �

� NGAXSNPB    PU PUTYPE=5,SUBAREA=194,TGN=1,                             - �

|                MACADDR=4000747000C2,SAPADDR=4                            �

|                                                                          �

� ����������������������Ŀ                                                 �

�Ĵ VMB � XCA MAJOR NODE �������������������������������������������������Ĵ

� ������������������������                                                 �

� NGBXCA VBUILD  TYPE=XCA                                                  �

� *                                                                        �

� NGBX280 PORT   CUADDR=280,ADAPNO=0,SAPADDR=4,MEDIUM=RING,              - �

|                DELAY=0.1,TIMER=20                                        �

� *                                                                        �

� *------------------------------------------------ NGBXCA SUBAREA NODES   �

� *---------------------------------------------------------------------   �

� *                                                                        �

� *--------------------------------------------------------- SUBAREA VMA   �

� NGBXSNGA GROUP DIAL=NO                                                   �

� *              ----------------------------------------------- LAN3172   �

� NGBXSNLA  LINE USER=SNA,ISTATUS=ACTIVE                                   �

� *                                                                        �

� NGBXSNPA    PU PUTYPE=5,SUBAREA=193,TGN=1,                             - �

|                MACADDR=4000747000C1,SAPADDR=4                            �

|                                                                          �

����������������������������������������������������������������������������

NOTES: (XCA >< XCA) (TYPE5 >< TYPE5) CONNECTION

. each P/390 has a TokenRing Adapter installed as LAPS ADAPTER NUMBER 0

. each VTAM in each P/390 is using SAP('04'x)

. each PU MACADDR= address is the destination (remote) LAN3172 LAN Network

Adapter Address (VTAM obtains the local LAN3172 LAN Network Address

by query from LAN3172).

. each PU SAPADDR= address is the destination (remote) LAN3172 SAP Address

(VTAM passes the local LAN3172 SAP Address (from PORT SAPADDR=) to

LAN3172 by command).

. PORT parameter DELAY= was added to VM by APAR VM58851 and will not be

valid without this VTAM APAR (added by PTF UV58746, 11/09/94)

��������������������������������Ŀ

�Ĵ VTAM XCA PERPHERIAL CONNECTION ���������������������������������������Ŀ

� ����������������������������������                                       �

� �����������������Ŀ                                                      �

� �  P/390          �                                                      �

� �����������������Ĵ                                                      �

� �  LAPS ADAPTER0  � �����������Ŀ                                        �

� ���������������Ŀ � �           �   ������������Ŀ                       �

� �� VMA   SA=193 ���Ĵ TokenRing ���Ĵ TYPE2.1 PU �  OS2/CM2 05D0000      |                 �

� ����������������� � �           �   ��������������                       �

� �  4000749000C1   � �������������                                        �

� �                 �                                                      �

� �                 �                                                      �

� �  LAPS ADAPTER1  � �����������Ŀ                                        �

� ���������������Ŀ � �           �   ������������Ŀ                       �

� �� VMA   SA=193 ���Ĵ EtherNet  ���Ĵ TYPE2.1 PU �  DOS/PC3270 05D0001   �

� ����������������� � �           �   ��������������                       �

� �  4000749001C1   � �������������                                        �

� �������������������                                                      �

� ���������������������������Ŀ                                            �

�Ĵ XCA MAJOR NODE /TOKENRING ��������������������������������������������Ĵ

� �����������������������������                                            �

� NGAXCAR VBUILD TYPE=XCA                                                  �

� *                                                                        �

� NGAX280R PORT  CUADDR=280,ADAPNO=0,SAPADDR=4,MEDIUM=RING,              - �

|                DELAY=0.1,TIMER=20                                        �

� *                                                                        �

� *--------------------------------------------- NGAXCA PERIPHERAL NODES   �

� *---------------------------------------------------------------------   �

� *                                                                        �

� NGAXRNG  GROUP DIAL=YES,CALL=INOUT,ANSWER=ON,ISTATUS=ACTIVE              �

� *                                                                        �

� NGBXRNL0 LINE                                                            �

� NGBXRNP0 PU                                                              �

�                                                                          �

� ��������������������������Ŀ                                             �

�Ĵ XCA MAJOR NODE /ETHERNET ���������������������������������������������Ĵ

� ����������������������������                                             �

� NGAXCAE VBUILD TYPE=XCA                                                  �

� *                                                                        �

� NGAX280E PORT  CUADDR=280,ADAPNO=1,SAPADDR=4,MEDIUM=CSMACD,            - �

|                DELAY=0.1,TIMER=20                                        �

� *                                                                        �

� *--------------------------------------------- NGAXCA PERIPHERAL NODES   �

� *---------------------------------------------------------------------   �

� *                                                                        �

� NGAXENG  GROUP DIAL=YES,CALL=INOUT,ANSWER=ON,ISTATUS=ACTIVE              �

� *                                                                        �

� NGBXENL0 LINE                                                            �

� NGBXENP0 PU                                                              �

�                                                                          �

� ���������������������Ŀ                                                  �

�Ĵ SWITCHED MAJOR NODE ��������������������������������������������������Ĵ

� �����������������������                                                  �

� NGASLAN VBUILD TYPE=SWNET                                                �

� *                                                                        �

� *        -  -- ---------------------------------------------- NGASXP00   �

� *                                                                        �

� NGASXPP0    PU PUTYPE=2,ADDR=C1,MAXDATA=2042,                          - �

�                IDBLK=05D,IDNUM=00000,CPNAME=NGASXP00,                  - �

�                DLOGMOD=HTPCMD2E,MODETAB=NGMODE,USSTAB=NGUSSS             �

� *                                                                        �

� NGASXL01    LU LOCADDR=1                                                 �

� NGASXL02    LU LOCADDR=2                                                 �

� NGASXL03    LU LOCADDR=3                                                 �

� NGASXL04    LU LOCADDR=4                                                 �

� *                                                                        �

� *        -  -- ---------------------------------------------- NGASXP01   �

� *                                                                        �

� NGASXPP1    PU PUTYPE=2,ADDR=C1,MAXDATA=1490,                          - �

�                IDBLK=05D,IDNUM=00001,CPNAME=NGASXP01,                  - �

�                DLOGMOD=HTPCMD2E,MODETAB=NGMODE,USSTAB=NGUSSS             �

� *                                                                        �

� NGASXL11    LU LOCADDR=1                                                 �

� NGASXL12    LU LOCADDR=2                                                 �

� NGASXL13    LU LOCADDR=3                                                 �

� NGASXL14    LU LOCADDR=4                                                 �

�                                                                          �

����������������������������������������������������������������������������

NOTES: (XCA PERIPHERAL) (TYPE2) CONNECTION

. single P/390 with two LAN Adapters installed

. a TokenRing Adapter installed as LAPS ADAPTER NUMBER 0

. an EtherNet Adapter installed as LAPS ADAPTER NUMBER 1

. two XCA MajorNodes (one for TokenRing, one for EtherNet)

. each XCA MajorNode references the same LAN3172 device (CUADDR=280)

. only one LAN3172 SubChannel address in DEVMAP (a single LAN3172 with

multiple LAN Adapters)

. each LINE/PU pair under the GROUP DIAL=YES requires one LAPS 802.2

link station (these link stations are allocated to LAN3172 when VTAM

activates the XCA MajorNode) so keep that in mind when deciding how

many LINE/PU pairs to code in the XCA MajorNode.

. note that MAXDATA= for EtherNet connections is limited to 1490 bytes

(EtherNet Adapters from vendors other than IBM may have a slightly

lower limit) -- a lower limit may imposed by the remote end depending

upon the EtherNet protocol stack and/or 3270 emulator used.

�����������������������������  ���������������������

�                           �  �                   �

� PERIPHERAL CONFIGURATIONS �  � LAN/802.2 LSApLAN �

�                           �  �                   �

�����������������������������  ���������������������

OVERVIEW

--------

From a peripheral point of view, the configuration at the peripheral

(remote) end of the connection is no different for a LAN3172 connection

than for a connection to a channel attached 3172.  Making peripheral

connections in installations having an existing 3172 should not be a

big deal.  However, peripheral configurations in environments without

a native 3172 can be a challenge, particularly for networked DOS/WINDOWS

computers whose 3270 connection is to a 3174 via coaxial cable (and not

presently integrated into the networking environment).

LAN3172 connections to networked DOS/WINDOWS computers usually require

that the LAN adapter be able to handle the SNA (LAN3172 3270/APPC) traffic

concurrently with that of the network.  The degree of difficulty, or

even whether or not it is even possible to handle the SNA frames, depends

upon what network operating system you are using, what kind of LAN adapter

you are using, and the protocol stacks available for your LAN adapter.

In general, the problem is easier for TokenRing than for EtherNet because

on TokenRing there is usually only one MAC frame format (which is usually

not the case with EtherNet with DIX, 802.3, etc).  Also, in general, it

is easier to configure with IBM/MicroSoft LAN Manager or Novell Netware

than with other network operating systems.

Usually, in the DOS/WINDOWS environment, sharing the LAN adapter between

the network and 3270/APPC applications means using an NDIS or ODI protocol

stack.  So without an NDIS or ODI driver for your LAN adapter, a concurrent

connection to the network and LAN3172 is unlikely.  Having an NDIS or ODI

driver is not enough for concurrent operation unless all of the concurrent

applications are able to interface to the NDIS or ODI stack.

For OS/2 (unlike DOS/WINDOWS) with a good LAPS NDIS implementation and

with virtual LAN drivers, sharing of the LAN adapter between applications

may be taken for granted.  Configuration of peripheral OS/2 connections

using either CM/2 or PC/3270 are straightforward.

PERIPHERAL PARAMETERS

---------- ----------

There are three (or four) parameters that the peripheral station must use

to establish an 802.2 session with LAN3172:

P/390 Gateway (LAN3172) Network Adapter Address

-----------------------------------------------

the 12 character hex Network Adapter Address (sometimes called the MAC

Address) of the P/390 LAN3172 Gateway.  This address is established during

LAPS configuration which manipulates the PROTOCOL.INI (adapter section)

file in the \IBMCOM SubDirectory on the boot drive.

for EtherNet adapters this address is configured via LAPS in Canonical

(ByteReversed) format.  However, when this address is configured on the

peripheral node, it is usually entered in NonCanonical (TokenRing or NOT

ByteReversed) format even when the peripheral node is using EtherNet.

P/390 Gateway (LAN3172) Network Adapter SAP Address

---------------------------------------------------

the 2 character hex Network Adapter SAP Address (sometimes called the

destination SAP address) of the P/390 LAN3172 Gateway.  This address is

defined by VTAM by the PORT SAPADDR= parameter:

���������������������������������������������������������������������Ŀ

� NGXCA VBUILD  TYPE=XCA                                              �

�                                                                     �

� NGX280E  PORT CUADDR=280,ADAPNO=1,SAPADDR=4,MEDIUM=CSMACD,          �

�               DELAY=0.1,TIMER=20                                    �

�����������������������������������������������������������������������

If the peripheral 802.2 application does not permit configuration of the

destination SAP (usually defaults to SAP('04'x) in this case), then the

VTAM XCA MajorNode must be configured to use this SAP.  Refer to the

SAP discussion at beginning of the VTAM CONFIGURATION section for more

information about the LAN3172 SAP value.

VTAM Switched Network PUID

--------------------------

an 8 character hex parameter (usually broken into a 3 character prefix and

a 5 character identification number) used to associate the peripheral node

to a specific entry in a VTAM Switched MajorNode:

��������������������������������������������������������������������������Ŀ

� NGASLAN VBUILD TYPE=SWNET                                                �

� *                                                                        �

� *        -  -- ---------------------------------------------- NGASXP00   �

� *                                                                        �

� NGASXPP0    PU PUTYPE=2,ADDR=C1,MAXDATA=2042,                          - �

�                IDBLK=05D,IDNUM=00000,CPNAME=NGASXP00,                  - �

�                DLOGMOD=HTPCMD2E,MODETAB=NGMODE,USSTAB=NGUSSS             �

� *                                                                        �

� NGASXL01    LU LOCADDR=1                                                 �

� NGASXL02    LU LOCADDR=2                                                 �

� NGASXL03    LU LOCADDR=3                                                 �

� NGASXL04    LU LOCADDR=4                                                 �

����������������������������������������������������������������������������

the Switched Network PU entry with a matching IDBLK= and IDNUM= contains

references to the peripheral node characteristics needed by VTAM to format

and interpret the data stream to and from the peripheral node.

VTAM CPNAME

-----------

VTAM will accept an 8 character Control Point name in lieu of a PUID. CM/2

is one of the 3270/APPC programs which supports the use of a CPNAME along

with or in place of a PUID (for CM/2 the CPNAME is the "Local node name"

in the "SNA local node characteristics" item for connections configured as

"Token-ring or other LAN types").  The Control Point name is the CPNAME=

value in the VTAM Switched MajorNode PU entry for the peripheral node:

��������������������������������������������������������������������������Ŀ

� NGASLAN VBUILD TYPE=SWNET                                                �

� *                                                                        �

� *        -  -- ---------------------------------------------- NGASXP00   �

� *                                                                        �

� NGASXPP0    PU PUTYPE=2,ADDR=C1,MAXDATA=2042,                          - �

�                IDBLK=05D,IDNUM=00000,CPNAME=NGASXP00,                  - �

�                DLOGMOD=HTPCMD2E,MODETAB=NGMODE,USSTAB=NGUSSS             �

� *                                                                        �

� NGASXL01    LU LOCADDR=1                                                 �

� NGASXL02    LU LOCADDR=2                                                 �

� NGASXL03    LU LOCADDR=3                                                 �

� NGASXL04    LU LOCADDR=4                                                 �

����������������������������������������������������������������������������

the Switched Network PU entry with a matching CPNAME= contains references

to the peripheral node characteristics needed by VTAM to format and

interpret the data stream to and from the peripheral node.

end LAN3172.DOC  17MAR1998/JLH