SCSI3420.SYS: 9-Track SCSI Tape Device Driver

=============================================

SCSI3420 is an OS/2 device driver which provides access to SCSI attached

3420 9-track reel-to-reel tape units from an IBM operating system running

on the P/390.  The 4mm DAT cartridge tape in the PC Server S/390 can also

be used to emulate a 3420 or 3422 tape.

DEVICE TYPE:  3420 or 3422

============

CONTENTS

========

A. Installation steps

B. Device Driver options

C. Data Compression

D. Trouble-shooting tips

E. Questions & Answers

F. Error messages

G. Supported tape drives

H. Printing this file

A. INSTALLATION STEPS

==================

These steps are for adding a SCSI tape drive to an IBM PC Server System/390.

The objective is to uniquely associate the tape drive with the device driver

(SCSI3420.SYS) which will control it for P/390 tape processing.

1. Set the SCSI ID of the tape drive.  The SCSI ID for a sequential device

may be a value from 0 to 6.  However, on a PC Server S/390, we suggest

using a SCSI ID from 1 to 6 in order to avoid possible conflict with the

4mm tape drive which has SCSI ID 0.  (For instructions about setting the

SCSI ID, refer to your tape drive user manual, or other documentation that

came with your tape drive).

2. Edit the CONFIG.SYS file.  It should contain the following statements:

BASEDEV=IBM2SCSI.ADD

(other ___.ADD drivers will go here)

BASEDEV=OS2SCSI.DMD

(other ___.DMD drivers will go here)

For a 9-track tape drive, uncomment the line:

DEVICE=d:\P390\SCSI3420.SYS  id,ai

"id" is the 2-digit hexadecimal SCSI ID of the tape drive (00 to 06)

Note: If the device is attached to a RAID adapter, then "id"

might be a "sequential device index", NOT the SCSI ID.

For instance, the first tape would be 01, second tape 02, etc.

The best way to determine what parameters to place on the

SCSI3480.SYS device driver is to use the /T option.

(See B. Device Driver Options, below).

"ai" is the Adapter Index (relative to origin 0) of the SCSI adapter

to which the tape drive is attached.  00 is the "first" adapter,

01 is the "second" adapter, and so on.

Note: Depending on the exact version of OS/2 you are running,

the adapter indices may not always follow the above rules!

The best way to determine what parameters to place on the

SCSI3420.SYS device driver is to use the /T option.

(See B. Device Driver Options, below).

Examples:

---------

A non-array PC Server 500 S/390 with the 4mm drive (SCSI ID 0) attached

to the SCSI controller in slot 1 and a SCSI 3480 drive (SCSI ID 2)

attached to a SCSI adapter in slot 3, would use the following

CONFIG.SYS statements:

DEVICE=d:\P390\SCSI3420.SYS  00,00  (internal 4mm)

DEVICE=d:\P390\SCSI3480.SYS  02,01  (external 3480)

In the RAID-array PC Server S/390, the 4mm DAT cartridge tape is attached

to the first RAID adapter and has SCSI ID 0, so you could use the

default statement in CONFIG.SYS:

DEVICE=d:\P390\SCSI3420.SYS  00,00

If you omit the "id,ai" parameters, the device driver will try to

allocate the highest numbered sequential device available on the lowest

slot SCSI adapter card (usually the 4mm drive).  Whenever you have more

than one active tape drive, you are encouraged to supply the "id,ai"

parameters.

Use "REM" to comment out the DEVICE statement for a device driver

which you do not want active.

3. Review the AWSCTYPE.MGR file.  It should contain an entry of the

following form for each active device driver/device type:

SCSI3420  3420

SCSI3480  3480

4. Review the IPL.CMD file.  It should contain a "dmstart" for

each active device driver, as follows:

call dmstart "awsdev.exe n /cu=SCSI3420 /dd=SCSI3420"

call dmstart "awsdev.exe n /cu=SCSI3480 /dd=SCSI3480"

5. Update the P/390 system devices.  Bring up the P/390 Configurator and

select "Update System Devices" (F2).  Enter the real address (used by

the S/390 Operating System) of the 3420 tape device in the Addr field

and the MGR code for SCSI3420 in the Mgr field.  The MGR code is found

in the Mgr Codes list at the bottom of the menu.

For example:

Addr  Device                         Mgr    FN/P

------------------------------------------------------------------

>582   >3420                         >G   >

Note: If you are running VM/ESA ESA feature, then specify the

device as a 3422. Otherwise the tape will not come online

automatically.

Steps 1-2 will take effect when you re-boot the system.  Steps

3-5 take effect at IPL P/390.

NOTE: Only ONE 3420 tape drive can be supported by SCSI3420 at a time.

Do not assign two tape devices to the same device manager. Do not

put the SCSI3420.SYS device driver in CONFIG.SYS more than ONCE!

There are TWO identical 3420 device managers called SCSI3420 and

AWS3420.  Thus, you can have up to FOUR simultaneous tape drives:

- SCSI3420  controlling a 3420 tape (or DAT emulating 3420)

- AWS3420   controlling a 3420 tape (or DAT emulating 3420)

- SCSI3480  controlling a 3480 tape (or DAT emulating 3480)

- AWS3480   controlling a 3480 tape (or DAT emulating 3480)

If you wish to use AWS3420, substitute this name everywhere that

"SCSI3420" is mentioned in this DOC file.

B. DEVICE DRIVER OPTIONS

=====================

Device driver options are activated by placing parameters at the end of

the device driver line in CONFIG.SYS.  Each of these options begin with

a forward slash (/) and must be separated from other options by a blank.

Option   Use

------   --------------------------------------------------------------

/T       Displays diagnostic tape device information during start-up,

showing which tape devices are available and confirming which

device was requested and allocated by SCSI3420.SYS.

CONFIG.SYS processing will be paused until you press ENTER.

This is a VERY useful option that should be used if you add an additional

SCSI adapter or tape device.  It will help you figure out what devices

the SCSI3420 device driver detects, and which parameters to put in

CONFIG.SYS.

Example

-------

When the device driver is started, the OS/2 start-up display will pause so

that you can view messages similar to those in the following example:

SCSI3420:  P/390 SCSI 9-TRK Tape Device Driver 1.21

Total sequential devices detected = 03.

Available tape unit = 00,00

Available tape unit = 05,01

Requested tape unit = 00,00

Allocated tape unit = 00,00

>> Press ENTER to continue ...

In the example above, a total of three tape drives are present (according

to OS/2).  Two drives are available:  SCSI ID 00 on the first adapter, and

SCSI ID 05 on the second adapter.  A third drive has apparently been

claimed by a previously started device driver.  The unit with SCSI ID 00 on

the first adapter was requested and successfully allocated.

Messages

--------

The tape information messages which you receive can be interpreted as follows:

No tape devices presented

OS/2 detected no sequential tape devices physically present.

Is the tape powered on?  Is the SCSI cable attached?

Total sequential devices detected = nn.

"nn" is the total number of tape units physically present on the

system according to OS/2.  This number includes both allocated

and available units.

Available tape unit = id,ai

One line of this form will appear for each tape unit which has

not already been allocated by another driver.

"id,ai" indicates the SCSI ID and Adapter Index of the unit.

>>>> No tape units available <<<<<

There are no tape units available for allocation.

Requested tape unit = id,ai

The DEVICE statement specified SCSI ID "id" on Adapter Index "ai".

Requested tape unit = blank

The DEVICE statement did not include the "id,ai" parameters.

Allocated tape unit = id,ai

The device driver successfully allocated this tape unit.

"id,ai" indicates the SCSI ID and Adapter Index of the unit.

"NONE." means that no tape drive was allocated.

C. DATA COMPRESSION

================

If the SCSI-attached tape drive is a real 9-track tape device, and this

tape drive supports both 1600 and 6250 bpi, then SCSI3420 will set the

density that is requested by the S/390 operating system.

If another tape drive, such as a 4mm DAT cartridge tape, is emulating a

3420, then the density requests from the S/390 operating system are

handled as follows:

- If 1600 bpi is requested, then hardware compression on the drive

is turned off.

- If 6250 bpi is requested, then hardware compression on the drive

is turned on.

If the SCSI-attached tape drive does not support hardware

compression or it is disabled, then an error is returned to the

S/390 operating system if 6250 is requested.

Some tape drives may have a switch to enable/disable hardware

compression; consult the drive's manual to make sure it is set on.

D. TROUBLE-SHOOTING TIPS

=====================

Here are some suggestions to help you investigate why a specific tape drive

is not being properly activated on an IBM PC Server 500 S/390.

1. Examine which SCSI IDs are configured in the PC Server:

a. Start the PC Server Reference program.

o For non-array PC Server systems, the Reference program

can be started by pressing the F1 key while the IBM BIOS logo

screen is displayed, just after booting the Server.

o For RAID-array PC Server systems, you must insert the Reference

diskette and boot the server from it.

b. Select "Set Configuration"

c. Select "View Configuration"

This will show what adapters are configured in which slots.

d. Select "Set and View SCSI Device Configuration"

This option is only valid if you have a non-RAID SCSI adapter

configured (you could have both).  Information about each SCSI device

will be displayed.  Tape drives appear as "Sequential Access" devices.

The Device Address (ID) is the SCSI ID of the tape drive.

Note that the RAID controller and any devices connected to it

will not appear in "Set and View SCSI Device Configuration".

If you have a tape drive connected to the RAID controller and

you want to see configuration information, do the following:

a. Shutdown OS/2 and reboot the Server with the IBM RAID Controller

Diskette inserted. (Do not press "F1".)

b. Select "Drive information"

The display will show, for example, that the 4mm drive is on

Channel 1, Bay 1.  Note that the RAID display does not show SCSI IDs.

The SCSI ID of a tape drive will be its Bay number minus 1.

Even though the RAID controller has two channels, they are viewed

as a single adapter with duplicate sets of device numbers.

DO NOT put two tape drives at the SAME SCSI ID on the same controller,

even on different channels. It is always better to assign UNIQUE SCSI

IDs to every tape drive, even across different controller cards.

2. Check which tape devices are detected by OS/2 at start-up:

- Edit CONFIG.SYS

- Add the /T parameter at the end of the statements for SCSI3420.SYS.

The resulting display will report the actual SCSI ID and Adapter Index

(in "id,ai" format) for each sequential device on the system.  After

you have discovered the true SCSI ID and Adapter Index for your

drive(s), modify the "id,ai" parameters on the DEVICE statements to

match (and remove the "/T").

See B. Device Driver Options, above, for more details on the  /T parameter.

NOTE:  On a RAID system, don't be surprised if the Adapter Index of a

tape drive is not what you expect.  More than one adapter may be

reported as Adapter Index 00, or the Adapter Index values may not be

consecutive.  This is why it is helpful to choose unique SCSI IDs for

the sequential devices on your system.

3. Does another OS/2 device driver (e.g. a tape backup utility) have

the SCSI tape device claimed?

Check your CONFIG.SYS to see if an OS/2 tape backup utility program

(such as SYTOS or NOVABACK) is active for the SCSI tape device. You

cannot have share the tape between the P/390 and an OS/2 program.

See Question & Answer #3, below.

4. Check for current copies of device drivers

Are the correct copies of the IBM2SCSI.ADD, OS2SCSI.DMD, SCSI3420.SYS,

SCSI3480.SYS files being accessed at start-up?  Do you have non-current

copies or multiple copies floating around in different directories?

Issue the OS/2 command "DIR d:\name.ext /S" on each drive to find

them all.

If you need a more current version, it can usually be downloaded

from the OS/2 Device Driver section of the IBM BBS (tel: 919-517-0001)

or via anonymous FTP from ftp.pcco.ibm.com

- Login as user=anonymous, password=your_id

- cd /pub/os2_drivers

- binary

- get filename.ext

5. To find out which P/390 device drivers are active and which devices

they currently control, type the following at an OS/2 prompt:

AWSSTAT /L

6. Under VM/ESA, a 3422 will come on-line automatically, but a 3420

device will not. To vary a 3420 on-line, type the following from

the VM/ESA system OPERATOR userid:

- SET RDEVICE 580 TYPE 3420 MODEL 8

- VARY ONLINE 580

E. QUESTIONS & ANSWERS

===================

Q1: Which device driver should I use for the 4mm DAT cartridge tape drive:

SCSI3420 or SCSI3480?

A1: The 4mm tape device's SCSI command set is more similar to that of a

3420 tape drive than a 3480, but either driver can be used for a

4mm tape device.  The device driver you select will determine the

personality of the tape drive (e.g, how it responds to the P/390

operating system).

When accessing a 4mm device as a 3420, a Mode Select for 6250 bpi will

invoke data compression, while requesting density 800 or 1600 bpi will

result in no data compression.  When accessing a 4mm device as a 3480, a

Mode Select for a 3480-XF will perform compression, while a Mode Select

for 3480 will do no data compression.

Q2: Do I need to keep the tape drive powered on all the time?

A2: The tape drive does need to be powered on when you boot the system, but

you can power it off and on as needed (to lower the noise level, etc.).

Q3: Can I have the same tape unit accessible to P/390 users and to

an OS/2 tape control program (e.g., an OS/2 back-up utility)

at the same time?

A3: No.  If the SCSI3420 device driver allocates a tape drive at OS/2

start-up, it has exclusive control of that device.  To start OS/2

with a specific tape drive not allocated for P/390 use, "REM" out

the DEVICE statement for that drive in CONFIG.SYS prior to the

next OS/2 start-up.

You can purchase a version of the device driver (ISITAPE) that

allows you to switch the tape drive between the P/390 and OS/2

without rebooting OS/2. Included with ISITAPE is an enhanced

version of XTAPE, an OS/2 backup program. See the bottom of

file d:\P390\XTAPELO.DOC for information on ordering this product.

(The following applies to Microchannel SCSI adapters only):

Another method is to define your hardware configuration with two

separate SCSI IDs, one for P/390 use and one for non-P/390 use, then

manally changing the SCSI ID setting on the tape drive prior to

booting the system.  For example, choose SCSI ID "2" for P/390 use

(matching the SCSI3420 DEVICE statement in CONFIG.SYS) and "3" for

non-P/390 use.  Perform the hardware "Set and view SCSI configuration"

two times:  once with the tape unit's SCSI ID set to "2", and once

with its SCSI ID set to "3".  Specify Presence Error Reporting

[DISABLED] and Not Present [KEEP].  Before booting the system,

manually set the SCSI ID on the tape drive to "02" to allocate it to

the P/390, or set it to "03" to allocate it to a non-P/390 process.

(The following applies to Microchannel SCSI adapters only):

Q4: I replaced my tape drive with a different model tape drive.  Both have

the same SCSI ID. The new tape drive doesn't work.

A4: You have to run the PC Server Reference program TWICE:

- Once with NO drive connected, to remove the old SCSI device.

- A second time with the new drive connected, to configure the new device.

The PC Server will not give you an error message (e.g. 174 error) because

the SCSI IDs have not changed.  But the model information that is

different will not be read until you reconfigure the device.

See Trouble Shooting Tip D.1, above, for how to get into the reference

program.  When you are in "Set and View SCSI Configuration" the first

time, be sure to change the Not Present �KEEP� to �REMOVE�, and then

press F10 Save.  Press F3 to exit back one level, connect the new SCSI

device, select "Set and View SCSI Configuration" again, and the new tape

drive should be detected.

F. ERROR MESSAGES

==============

1. If you receive the following "pop-up" message:

+-------------------------------------------+

|                P/390 Error                |

+-------------------------------------------+

| AWSDEV908E CU=SCSI342: RC=006E            |

| opening device driver, DD=SCSI3420        |

+-------------------------------------------+

this means that you need to activate the SCSI3420.SYS device

driver in CONFIG.SYS.  You have put a 3420 device entry in DEVMAP,

and specified SCSI3420 as the device manager (Mgr code = SCSI3420).

Edit CONFIG.SYS and make sure the "rem" characters are deleted

from this line:

rem DEVICE=d:\P390\SCSI3420.SYS id,ai

You MUST specify the appropriate numbers for id (device ID) and ai

(adapter index) as discussed above in "Configuring a SCSI3420".

The /T option will prove useful at this time.

Reboot OS/2 and look for the SCSI3420 message during boot.

If you are temporarily not using the tape drive and want to get rid

of the error message, you can "exclude" the device from DEVMAP.

Go to the P/390 Configurator "F2 Update System Devices" menu and place

the cursor under the device address.  Press Alt+F3 to exclude the

device.  See the online Configurator help or the "PC Server S/390

Installation, Configuration, and User's Guide for VM/ESA" for more

details and examples.

2.   SYS1201: The device driver d:\P390\SCSI3420.SYS specified on line

nn of CONFIG.SYS was not installed. Line nn ignored.

This message is issued by OS/2 whenever there is no tape device

detected at the id,ai parameters as specified on the SCSI3420.SYS

statement in CONFIG.SYS.

o If this OS/2 error was preceded by the message:

** SCSI3420 OS2SCSI No tape devices presented

Then check for the following problems:

- Tape drive is powered off

- Tape drive SCSI cable is disconnected

o If this OS/2 error was preceded by the message:

** SCSI3420 OS2SCSI Open Interface error

Then check to see if CONFIG.SYS contains the following lines:

BASEDEV=IBM2SCSI.ADD

.   .   .

BASEDEV=OS2SCSI.DMD

o Otherwise, check the tape drive's SCSI ID

- For non-RAID SCSI adapters, run PC Server Reference diskette

- If the tape drive is attached to the RAID SCSI adapter, run the

PC Server RAID option diskette, menu selection "Drive information".

In any case, try adding the /T parameter at the end of the line in

CONFIG.SYS. This will cause additional diagnostic information to be

displayed when you reboot OS/2.

3. Details on AWSDEV errors when used with the tape device drivers

---------------------------------------------------------------

There are two possible AWSDEV initialization-time errors that can

occur due to errors returned from the tape device driver.  In addition,

there is one other "common" AWSDEV error that generally results from

problems with the device driver.

These will be displayed by an error "pop-up" box and will be written

to AWSERROR.LOG.  The error messages are of the form:

AWSDEV908E CU= $s: RC=$x opening device driver, DD= $s

AWSDEV909E CU= $s: RC=$x issuing "DD_init" order, DD= $s

AWSDEV912E CU= $s: RC=$x creating the VCU queue

Substitutable variable text will be provided by AWSDEV for the

"CU=" and "DD=" fields to identify the CU instance and DD to which

the error applies.  With the current IPL.CMD, this text will be

the same for "CU=" and "DD=" and will be one of:

(1) SCSI3480

(2) SCSI3420

(3) AWS3480

(4) AWS3420

(5) ISITAPE

This is intended to assist in identifying the device(s) which are the

subject of the error.  Use the configurator to identify which device(s)

are using these CU names.  The CU names are assigned a letter by the

configurator at the bottom of the "F2" "Update System Devices" screen.

The "RC=" field provides a specific reason for the error.  The following

are some "common" errors, RCs, and their meanings.  The device driver

return codes are from a cursory code review of SCSI3480 and do not

necessarily apply to the other drivers listed above (although it is

believed all of the above drivers are similar).  RCs of the form "FFxx"

originate from the device driver, not AWSDEV.  While the device driver

can return errors other than those listed below, only these should

result in AWSDEV error pop-up messages.  The other errors either result

in P/390 trace entries and would require an I/O trace to analyze or

would be the result of major incompatibilities/bugs between AWSDEV and

its device driver(s).  That is, the other errors are not expected to

be "common".

AWSDEV908E (Error during device driver open by AWSDEV):

------------------------------------------------------------------------

RC = 006E  Open Failed

The most likely explanation is that the listed device driver

was not loaded during OS/2 system initialization.  Check

CONFIG.SYS and be sure a valid device driver invocation

is present.

RC = FF05  Unknown / unrecognized tape device

Device driver does not support or recognize the tape drive

it has been assigned to use.

AWSDEV909E (Error during device driver IOCTL initialization by AWSDEV):

------------------------------------------------------------------------

RC = FF01  No tape device allocated

The device driver was not able to allocate the tape device

during OS/2 system initialization.

RC = FF05  Too many tape devices assigned to the device driver

The device driver found that more devices were assigned to

it than it can support.  For SCSI3480, SCSI3420, AWS3480,

and AWS3420 only one device can be assigned.  By loading

two x3480 device drivers (SCSI3480 and AWS3480) or two

x3420 device drivers (SCSI3420 and AWS3420), an aggregate

of two devices of the indicated type can be supported in

the system.

RC = FF0C  Unable to map P/390 trace area to GDT entry

An error occurred during an OS/2 PhysToGDTSelector call

which is needed to provide the device driver with

addressability to the P/390 trace area.

AWSDEV912E (Error during CU request queue creation):

------------------------------------------------------------------------

RC = 014C  Duplicate OS/2 queue name

AWSDEV was not able to create the queue needed to field

requests from the P/390 channel.  This normally indicates

a previous AWSDEV/device driver instance was not completely

terminated.  When the P/390 is shutdown, the AWSDEV process

must end.  If any of its threads are not freed (to die),

it will not complete its termination and its OS/2 queue

will remain.  Note that this incomplete termination is not

obvious until the *next* time the P/390 is started and this

error given.  It is believed the OS/2 SCSI device driver

used by the x3420 and x3480 device drivers can get "stuck"

and not properly respond to the process termination signal.

This causes an AWSDEV thread to be held in the OS/2 kernel

and prevents the process termination.  When this happens,

the only known solution is to reboot OS/2.

G. Supported tape drives

=====================

The following SCSI tape drives are currently supported by SCSI3420.SYS:

(Additional drives are supported by ISITAPE.SYS. See XTAPELO.DOC for details)

4mm

---

- IBM  4/10 GB DDS-2 (74G8631 or 74G8632)

- IBM 12/24 GB DDS-3 (01K1282)

9-track

-------

- M4 Data 9914, 9906

The following tape drives are only supported on older PC Server S/390 systems;

they are NOT supported by SCSI3420 on the PC Server S/390 Model 330 8640-PB0:

9-track

-------

- Overland Data 5622

- Qualstar 3414, 3418

Notes:

-----

1. When you are purchasing a SCSI-attached 3420 or 3480 tape drive,

make sure the drive has been tested by the dealer with the P/390

SCSI tape device drivers. There are different levels of hardware and

firmware, usually found in older or refurbished drives, that will

not operate correctly or at all with the P/390.

2. SCSI-attached 3420 and 3480 tapes drives (including the 4mm DAT cartridge)

are only supported when attached to these cards:

PC Server 500 or 520:

--------------------

Name in Reference Diskette Configuration  Adapter ID  Ordering P/N

----------------------------------------  ---------- -------------

IBM PS/2 SCSI-2 Fast/Wide Adapter/A             8EFC   70G8498

PC Server 330:

-------------

Product Name                              Feature Code  P/N

----------------------------------------- ------------ ---------

IBM PC ServeRAID Adapter                   1930         70G8489

IBM PC ServeRAID II Ultra SCSI Adapter                  76H3584

IBM PC Server Ultra Wide SCSI PCI Adapter  8260         76H5407

If you have two different SCSI adapters installed in the same PC, then you

may need an updated OS2SCSI.DMD driver, available from the P/390 FTP site:

ftp://P390.IBM.COM/P390/OS2/OS2SCSI.ZIP

3. The P/390 I/O subsystem does not support the READ BACKWARDS CCW unless

the SKIP bit is on.  This may affect certain tape utility programs.

It applies to all tapes, whether emulated, SCSI-attached, or Bus &

Tag Channel-attached via the S/370 Channel Emulator/A adapter.

This restriction does NOT apply to the S/390 Parallel Channel Adapter.

4. Make sure that the SCSI bus is properly terminated. For instance,

a wide SCSI adapter requires WIDE termination, even if the tape

drive you are attaching is narrow SCSI.

H.  Printing this file

==================

If you are viewing this file on-line via the P/390 Configurator, be

aware that this (and all other P/390 device manager DOC files) are

plain ASCII text files that can be printed on your PC printer.

Look in the d:\P390 subdirectory for all *.DOC files (where d: is the

drive where you installed the P/390 Program).

You are currently viewing d:\P390\SCSI3420.DOC.

---------------------------------------------------------------------------

The information in this document was developed in cooperation

with the SCSI3420 device driver developers.