AWSSAD: System Activity Display

===============================

AWSSAD creates a presentation manager window that shows the S/390

processor activity.  This is shown by a sliding bar (sometimes called a

"speedometer") and a display of the instruction address portion of the

PSW (program status word).

Internally AWSSAD is treated as a device manager, even though it does

not have any S/390 devices associated with it.  It is started from

IPL.CMD and you can pass certain parameters to it that affect the

appearance of its window.

Colors in System Activity Display Window

========================================

The colors in the "speedometer" indicate how the S/390 processor is

being used.  The amount of each color represents the percentage of CPU

time spent in that mode during the last interval.  The display is

updated approximatly once every 150 milliseconds.

Blue   - Wait State

Red    - Supervisor State

Green  - Problem State

Pink   - CPU waiting for I/O initial status

In addition there is an optional yellow bar, approximately 1/3 the

height of the CPU bar that shows I/O activity.

Yellow - SIO/SSCH instructions per second

This yellow bar is present only of the /SIO nnn option is specified as

an AWSSAD parameter.  "nnn" represents the maximum number of starts per

second expected -- i.e. when the bar is solid yellow, the P/390 is

executing at least that many starts per second.

A solid green bar generally means you're getting lots of application run

time.  A solid red bar means the supervisor is very busy.  A solid blue

bar changes to the text "waiting" if the processor remains idle very

long.  A solid pink bar means the processor is waiting on the PC to

accept or reject an I/O request.  If this remains for a long time it

becomes a "pink hang" and generally means there is something wrong with

the P/390 I/O subsystem.  You should take a trace and a snap dump and

call for service if this happens.

Text in the System Activity Display Window

==========================================

When the processor is not in the run state, the color bar is replaced

with text that indicates the processor state:

Stopped:  The processor is in a normal stopped state.  IPL or press START

in MANOPS (P/390 Manual Operations) may be used to restart it.

If the P/390 enters a disabled wait state, the processor

status changes to "stopped" and the PSW displays the

wait state code which the operating system uses to

indicate why it stopped.

Loading:  This is normally a transient state during IPL.  After

IPL, "loading" appears briefly while the first channel

program is executed.  Then the color bar appears and

the system IPL proceeds.  If "loading" remains and doesn't

go away, there is a problem with the IPL device.  Check

to make sure you are IPLing the right device, and turn on

tracing to get a clue as to why the IPL failed.

Chkstop:  This state is entered after a CPU machine check.  You may

be able to resume execution by invoking MANOPS and trying

a CPU reset and then a START.  ***BUT*** this may not be

wise since damage to data in storage might have occurred.

Waiting:  If the CPU is in wait state or in key 3 active wait,

the blue bar is replaced by "waiting" after a few seconds.

Parameters in IPL.CMD

==============================

call dmstart "awsSAD.exe   p /SIO 300" dmparms.awssad;

This is the statement in IPL.CMD that starts AWSSAD.  As mentioned

before, the /SIO 300 indicates that the maximum I/O rate is expected to

be 300 starts per second.  You may need to adjust this value up or down

depending on your system.  If the yellow bar is frequently "pegged" then

increase the parmameter to get a better idea of what rate you are

actually achieving.

Other parameters can also be specified:

/XS xxx    Width of SAD display

/YS yyy    Height of SAD display

/FLOAT     Keep the SAD display visible at all times

/XS and /YS let you fine-tune the size of the AWSSAD window.  Depending

on the monitor, font, and display adapter used, you may find the PSW to

be too large or too small.  Playing with /XS and /YS may improve the

appearance.  The values are usually in the 100 to 400 range.

/FLOAT keeps the SAD window "floating on top" of other windows, but

without having the keyboard focus.  This way the S/390 activity is always

visible and doesn't get hidden behind other windows on the screen.