A 68000 Assembler

----------------

This assembler is used to define words accessible from forth in assembly

language.  It is used for those functions that cannot be implemented

efficiently enough in forth.  It is usually most productive to develop a

solution entirely in forth, and then recode inefficient functions in assembly

language.

The syntax of the assembler follows the forth convention of putting operands on

the stack before the assembly language instruction.  This allows the user to

take advantage of the full power of forth to compute values, do macros, and do

conditional assembly.

Writing Code Words

------------------

The code words that correspond to ':' and ';' are 'code' and 'end-code'.

'Code' saves the current computation stack depth (for later error checking in

'end-code'), creates the code word header containing the name of the code word

and various other stuff, and leaves the system ready for the following assembly

language instuctions.  When the code word is done, normally the 'next' macro is

used followed by 'end-code'.  The 'next' macro assembles the instructions

needed to execute the next forth word.

code word example:

------------------

code 0=			\ define the code word '0='

s ) tst		\ test the top number on the computation stack

ze if		\ is it equal to zero?

1 # s ) move	\ if yes, replace it with 'true'

else

0 # s ) move	\ if not, replace it with 'false'

endif

next		\ execute the next forth word

end-code		\ end the code word

The code word that corresponds to 'does>' is ';code'.  It is used in defining

words that need to be written in assembly language instead of forth.

';code' example:

----------------

: constant		\ define the compiling word 'constant'

create		\ create the header for a constant

smudge		\ toggle the smudge bit so the header can be found

,			\ deposit the constant value

;code			\ define the action of the constant in code

w ) s -( move	\ move the constant value to the computation stack

next		\ execute the next forth word

end-code		\ end the defining word

condition codes and code control structures

-------------------------------------------

equality comparisons

--------------------

eq	equal

ne	not equal

unsigned comparisons 		signed comparisons

--------------------		------------------

hi	high 			ge	greater than or equal

hs	high or same 		gt	greater	than

lo	low 			le	less than or equal

ls	low or same 		lt	less than

status bit comparisons

----------------------

cc	carry clear 		vc	overflow clear

cs	carry set 		vs	overflow set

pl	plus (negative clear) 	nz	non-zero (zero clear)

mi	minus (negative set) 	ze	zero set

The condition codes are used as inputs to the code control structures and the

assembly language code words 'bcc', 'lbcc', 'dbcc' and 'scc'.

The code control structures are as follows:

begin		\ Execute the code between 'begin' and 'again'

...		\  in an infinite loop.

again

begin		\ Execute the code between 'begin' and 'until'

...		\  until the condition code 'cc' is true.

cc until

begin		\ Execute the code between 'begin' and 'repeat'

...		\  while the condition code 'cc' is true.  The loop is

cc while	\  left at the 'while' once the condition code is false.

...

repeat

cc if		\ If the condition code 'cc' is true, then execute the

...		\ code between 'if' and 'endif'.

endif

cc if		\ If the condition code 'cc' is true, then execute the

...		\ code between 'if' and 'endif'.  If the condition code

else		\ is false, then execute the code between 'else' and

...		\ 'endif'.

endif

These code control structures have the same names as the corresponding forth

control structures.  They perform the same functions for code words as the

corresponding forth control structures do for forth words.  When you are in

the 68kasm vocabulary (such as when you are defining a code word) they are

found instead of the forth control structures.

These code control structures are based on 8-bit relative offsets.  An 8-bit

offset is adequate for most code words, but if you get an 'out of range' error

while assembling a code word, there are a corresponding set of code control

structures based on 16-bit relative offsets available.  These have the same

names as the 8-bit code control structures except that an 'l' is added to the

front of the name.

The assembly language words 'bcc' and 'lbcc' are normally not needed, since the

code control structures provide a higher-level (better) solution to normal

branching and looping requirements.  If you want to use them, you must provide

the branch address yourself.

Due to the limitation of 'dbcc' to a 16-bit value, I don't use it.  I have

not made any effort to provide higher-level access to it.  If you want to use

it, you must provide the branch address yourself.

register names and usage

------------------------

68000	forth

reg	name	comment

----------------------------------------------------------------------------

a7	rp	reserved for use as the 'return stack pointer' register and the

HP-UX stack pointer register

a6		reserved for use as the HP-UX stack frame pointer register

a5	s	reserved for use as the 'computation stack pointer' register

a4	ip	reserved for use as the 'instruction pointer' register

a3	w	used in 'next'; may be used as a scratch register in code words

a2		used in 'next'; may be used as a scratch register in code words

a1		may be used as a scratch register in code words

a0		may be used as a scratch register in code words

d7		may be used as a scratch register in code words

d6		may be used as a scratch register in code words

d5		may be used as a scratch register in code words

d4		may be used as a scratch register in code words

d3		may be used as a scratch register in code words

d2		may be used as a scratch register in code words

d1		may be used as a scratch register in code words

d0		may be used as a scratch register in code words

instruction syntax

------------------

<source operand or EA> <destination operand or EA> <operand size> <opcode>

One or both of the operand fields may be absent.  Any source operand or

effective address (EA) must occur before any destination operand or effective

address.

The operand size field may be absent.  The possible values for the operand size

field are 'byte', 'word', and 'long' corresponding to 8 bits, 16 bits and 32

bits.  If the operand size field is absent the operand size defaults to 'long'.

effective addressing modes

--------------------------

addressing mode				corresponding 68kasm syntax

-----------------------------------------------------------------------------

data register direct					   <d0-d7>

address register direct					   <a0-a7>

address register indirect				   <a0-a7>   )

(An) with postincrement					   <a0-a7>   )+

(An) with predecrement					   <a0-a7>   -(

(An) with displacement		<n16> 			   <a0-a7>   d(

(An) with short index		<n8>	<d0-d7 or a0-a7>   <a0-a7>   di.w(

(An) with long index		<n8>	<d0-d7 or a0-a7>   <a0-a7>   di(

absolute short address		<n16> 				     (#.w)

absolute long address		<n32> 				     (#)

(PC) with displacement		<n16> 				     d(pc)

(PC) with short index		<n8>	<d0-d7 or a0-a7> 	     di.w(pc)

(PC) with long index		<n8>	<d0-d7 or a0-a7> 	     di(pc)

immediate data			<n*>				     #

* 	The size of immediate data must match the operand size.

-----------------------------------------------------------------

|		effective addressing mode categories		|

|---------------------------------------------------------------|

| 				| data  |memory |control| alter-|

| addressing mode		|	|	|	| able  |

|---------------------------------------------------------------|

| data register direct		|   X	|   -	|   -	|   X	|

|-------------------------------+-------+-------+-------+-------|

| address register direct	|   -	|   -	|   -	|   X	|

|-------------------------------+-------+-------+-------+-------|

| address register indirect	|   X	|   X	|   X	|   X	|

| addr reg ind w/ postincrement	|   X	|   X	|   -	|   X	|

| addr reg ind w/ predecrement	|   X	|   X	|   -	|   X	|

| addr reg ind w/ displacement	|   X	|   X	|   X	|   X	|

| addr reg ind w/ short index	|   X	|   X	|   X	|   X	|

| addr reg ind w/ long index	|   X	|   X	|   X	|   X	|

|-------------------------------+-------+-------+-------+-------|

| absolute short address	|   X	|   X	|   X	|   X	|

| absolute long address		|   X	|   X	|   X	|   X	|

|-------------------------------+-------+-------+-------+-------|

| prog counter w/ displacement	|   X	|   X	|   X	|   -	|

| prog counter w/ short index	|   X	|   X	|   X	|   -	|

| prog counter w/ long index	|   X	|   X	|   X	|   -	|

|-------------------------------+-------+-------+-------+-------|

| immediate data		|   X	|   X	|   -	|   -	|

-----------------------------------------------------------------

source operand or EA		dest operand or EA	size	instruction

-------------------------------------------------------------------------------

Dy				Dx		b	abcd

Ay	-(			Ax	-(	b	abcd

<EA>					Dn		b w l	add

Dn			<memory alterable EA>	b w l	add

<EA>					An		  w l	adda

n		#		<data alterable EA>	b w l	addi

n(1-8)		#		<alterable EA>		b w l	addq

Dy				Dx		b w l	addx

Ay	-(			Ax	-(	b w l   addx

<data EA>				Dn		b w l	and

Dn			<memory alterable EA>	b w l	and

n		#		<data alterable EA>	b w l	andi

n		#					b	andi>ccr

n		#					  w	andi>sr

Dx				Dy		b w l	asl

n(1-8)		#			Dy		b w l	asl

<memory alterable EA>	  w	asl

Dx				Dy		b w l	asr

n(1-8)		#			Dy		b w l	asr

<memory alterable EA>	  w	asr

<branch addr>		<condition code>	b	bcc

Dn			<data alterable EA>	b   l	bchg

n		#		<data alterable EA>	b   l	bchg

Dn			<data alterable EA>	b   l	bclr

n		#		<data alterable EA>	b   l	bclr

Dn			<data alterable EA>	b   l	bset

n		#		<data alterable EA>	b   l	bset

Dn			<data alterable EA>	b   l	btst

n		#		<data alterable EA>	b   l	btst

<branch addr>					b	bra

<branch addr>					b	bsr

<data EA>				Dn		  w	chk

<data alterable EA>	b w l	clr

source operand or EA		dest operand or EA	size	instruction

-------------------------------------------------------------------------------

<EA>					Dn		b w l	cmp

<EA>					An		  w l	cmpa

n		#		<data alterable EA>	b w l	cmpi

Ay	)+			Ax	)+	b w l	cmpm

Dn	<branch addr>		<condition code>	  w	dbcc

Dn	<branch addr>					  w	dbra

<data EA>				Dn		  w	divs

<data EA>				Dn		  w	divu

Dn			<data alterable EA>	b w l	eor

n		#		<data alterable EA>	b w l	eori

n		#					b	eori>ccr

n		#					  w	eori>sr

Dx				Dy		    l	exg

Ax				Ay		    l	exg

Dx				Ay		    l	exg

Dn		  w l	ext

illegal

<control EA>			jmp

<control EA>			jsr

<branch addr>		<condition code>	  w	lbcc

<branch addr>					  w	lbra

<branch addr>					  w	lbsr

<control EA>				An		    l	lea

n		#			An			link

Dx				Dy		b w l	lsl

n(1-8)		#			Dy		b w l	lsl

<memory alterable EA>	  w	lsl

Dx				Dy		b w l	lsr

n(1-8)		#			Dy		b w l	lsr

<memory alterable EA>	  w	lsr

source operand or EA		dest operand or EA	size	instruction

-------------------------------------------------------------------------------

<EA>				<data alterable EA>	b w l	move

<EA>					An		  w l	movea

<register list mask>		<control or -( EA>	  w l	movem>

<control or )+ EA>		<register list mask>	  w l	movem<

Dx			n	Ay	d(	  w l	movep

n	Ay	d(			Dx		  w l	movep

n8		#			Dn		    l	moveq

<data EA>						  w	move>ccr

<data EA>						  w	move>sr

An						    l	move>usp

<data alterable EA>	  w	move<sr

An		    l	move<usp

<data EA>				Dn		  w	muls

<data EA>				Dn		  w	mulu

<data alterable EA>	b	nbcd

<data alterable EA>	b w l	neg

<data alterable EA>	b w l	negx

nop

<data alterable EA>	b w l	not

<data EA>				Dn		b w l	or

Dn			<memory alterable EA>	b w l	or

n		#		<data alterable EA>	b w l	ori

n		#					b	ori>ccr

n		#					  w	ori>sr

<control EA>						    l	pea

reset

source operand or EA		dest operand or EA	size	instruction

-------------------------------------------------------------------------------

Dx				Dy		b w l	rol

n(1-8)		#			Dy		b w l	rol

<memory alterable EA>	  w	rol

Dx				Dy		b w l	ror

n(1-8)		#			Dy		b w l	ror

<memory alterable EA>	  w	ror

Dx				Dy		b w l	roxl

n(1-8)		#			Dy		b w l	roxl

<memory alterable EA>	  w	roxl

Dx				Dy		b w l	roxr

n(1-8)		#			Dy		b w l	roxr

<memory alterable EA>	  w	roxr

rte

rtr

rts

Dy				Dx		b	sbcd

Ay	-(			Ax	-(	b	sbcd

<data alterable EA>		<condition code>	b	scc

n		#					  w	stop

<EA>					Dn		b w l	sub

Dn			<memory alterable EA>	b w l	sub

<EA>					An		  w l	suba

n		#		<data alterable EA>	b w l	subi

n(1-8)		#		<alterable EA>		b w l	subq

Dy				Dx		b w l	subx

Ay	-(			Ax	-(	b w l   subx

Dn		  w	swap

<data alterable EA>	b	tas

n(0-15)		#						trap

trapv

<data alterable EA>	b w l	tst

An			unlk